# GNN-DQN Handover Optimization Project

A-to-Z Technical Guide in Simple Language

Prepared for thesis defense and professor Q&A. Generated from the local repository on May 13, 2026. Main path: UE_ONLY features + SON-GNN-DQN deployment framing.

One-sentence idea The model watches what a UE can measure from nearby cells, uses a graph neural network to score handover targets, and then a safety-bounded SON controller turns those learned preferences into normal A3/CIO/TTT-style handover behavior.

# How To Read This Document

This guide is written in simple language first, with technical details immediately after. The analogy is: imagine every cell tower is a stop in a city bus network, every UE is a passenger moving through the city, and handover is choosing which bus stop should serve the passenger next. A bad handover is like making the passenger jump between stops again and again; a good handover keeps the passenger connected, avoids crowded stops, and does not make unnecessary jumps.

If you need the defense story, read Sections 1, 2, 8, 11, and 15.

If you need implementation detail, read Sections 3 to 10.

If you need professor questions, read Section 16.

If you need commands, read Section 14.

# 1. Project Outline

## Main Goal

The project builds a defense-ready handover optimization framework for LTE/5G-style cellular networks. The final deployable claim is not simply “an AI directly controls handover.” The stronger and safer claim is:

Deployable claim A topology-generalized GNN-DQN learns per-UE handover preferences from UE-observable measurements, then a safety-bounded SON layer translates those preferences into standard A3/CIO-style handover parameter updates.

In simplest language: the AI recommends where users should prefer to move, but the network still uses familiar telecom rules to make the final handover. This is easier to defend because real networks do not allow unsafe black-box handover commands.

## Primary Direction

| Decision | What it means | Why it matters |
| --- | --- | --- |
| Main feature profile | ue_only | Uses features a UE or drive-test can observe, such as RSRP and RSRQ. This is practical for thesis validation. |
| Main deployable method | son_gnn_dqn | The learned model feeds a SON controller that updates bounded CIO/TTT values. |
| Direct GNN-DQN | gnn_dqn baseline | Useful to show the learned preference model, but not the main deployment claim. |
| O-RAN/E2 path | future/demo profile | Can later add true PRB utilization and network KPIs, but should not be the headline. |
| Missing PRB in drive-test data | prb_utilization = 0 and prb_available = 0 | Keeps the data honest; RSRQ proxy is not falsely called real PRB. |

## What Problem Is Being Solved?

A UE can usually hear several nearby cells. The network must decide whether the UE should stay on the current serving cell or hand over to another cell. A simple strongest-signal rule can overload popular cells. A very aggressive handover rule can create ping-pong, where the UE bounces between cells. This project tries to balance signal quality, load, fairness, outage avoidance, and handover stability.

# 2. Project Tree Structure

The active repository is organized so code, configs, outputs, and thesis writing are separate.

/Users/saangetamang/gnn-dqn-handover-loadbalancing
|-- configs/experiments/        JSON configs for smoke, diagnostic, defense, and full runs
|-- configs/scenarios/          Scenario documentation
|-- data/raw/                   OpenCellID, ns-3, drive-test, and raw input data
|-- data/processed/             Generated/cleaned data, ignored by git
|-- docs/                       Guides, thesis chapters, reports, defense notes
|   |-- guides/                 Human-readable guides and DOCX exports
|   |-- thesis/                 Thesis chapters and abstract
|   |-- reports/                Result and evaluation reports
|-- scripts/                    Config-driven entrypoints: train, evaluate, figures, summaries
|-- src/handover_gnn_dqn/       Reusable Python package
|   |-- env/                    LTE/5G simulator, state, reward, mobility, metrics
|   |-- models/                 GNN-DQN and Flat-DQN model code
|   |-- rl/                     Training loop, replay, validation, checkpointing
|   |-- policies/               Baselines: A3, strongest RSRP, load-aware, random, GNN, SON
|   |-- son/                    Safety-bounded SON controller
|   |-- topology/               Synthetic and real topology/scenario builders
|-- tests/                      Unit and integration acceptance tests
|-- results/archive_prefix/     Old pre-refactor diagnostics, not final evidence
|-- results/runs/               New generated experiment runs, ignored by git
|-- web_dashboard/              Optional local visualization/dashboard

## Important Files

| File | Role |
| --- | --- |
| scripts/train.py | Loads a JSON config, selects scenarios, trains the GNN-DQN, writes checkpoints and history. |
| scripts/evaluate.py | Loads a checkpoint and evaluates policies over many random seeds. |
| src/handover_gnn_dqn/env/simulator.py | Simulates cells, UEs, RSRP, RSRQ, load, throughput, mobility, reward, and metrics. |
| src/handover_gnn_dqn/models/gnn_dqn.py | Defines GNN-DQN model, replay buffers, n-step returns, batched Q-learning update. |
| src/handover_gnn_dqn/rl/training.py | Multi-scenario training, behavioral cloning warm start, validation, checkpoint logic. |
| src/handover_gnn_dqn/son/controller.py | Converts learned GNN preferences into bounded CIO/TTT updates. |
| src/handover_gnn_dqn/policies/policies.py | Classical baselines and learned policies used in evaluation. |
| configs/experiments/multiscenario_ue.json | Clean 300-episode UE-only production run. |
| configs/experiments/multiscenario_ue_defense.json | Longer defense run design: BC warm start, PER, n-step, validation, curriculum. |

# 3. Wireless Background In Simple Language

## Key Terms

| Term | Plain meaning | In this project |
| --- | --- | --- |
| UE | User equipment, like a phone or modem. | Each UE moves, measures nearby cells, and chooses a serving cell. |
| Cell | A base-station sector or tower coverage point. | A graph node that can serve UEs. |
| Handover | Moving a UE from one serving cell to another. | The model action is the target cell index. Staying is also an action. |
| RSRP | Reference Signal Received Power: signal strength. | Used to know which cells are strong enough. |
| RSRQ | Reference Signal Received Quality: quality affected by interference/load. | Used as a UE-observable load/interference proxy, not real PRB. |
| PRB | Physical Resource Block, a real radio resource counter. | Only used in O-RAN/E2 profile when actually available. |
| CIO | Cell Individual Offset. | SON changes CIO to make a target cell slightly more or less attractive. |
| TTT | Time To Trigger. | SON changes how many steps a condition must remain true before handover. |
| Ping-pong | UE quickly returns to the previous cell. | Penalized because it wastes signaling and hurts stability. |

## A3 Handover Rule

The classical A3 rule says: hand over when a neighbor becomes better than the serving cell by a margin, and stays better for enough time.

A3 condition:
RSRP_target + CIO(source,target) > RSRP_serving + A3_offset

Only hand over if this stays true for TTT steps.

Analogy: do not switch bus stops just because another stop looks slightly better for one second. Wait for it to be clearly better and stable.

## Wireless Equations Used By The Simulator

Distance in km:
d_km = max(distance_m / 1000, 0.035)

Path loss:
PL_dB = 128.1 + 37.6 * log10(d_km)

Received signal strength:
RSRP_dBm = TxPower_dBm - PL_dB + shadowing + noise

Quality from RSRP:
quality = sigmoid((RSRP + 96) / 7), clipped to [0.04, 1.0]

Cell load pressure:
pressure_ue = demand_mbps / max(quality, quality_floor)
load_cell = sum(pressure_ue served by cell) / cell_capacity_mbps

Throughput:
throughput_ue = demand_ue / max(load_serving_cell, 1)
If the serving RSRP is below threshold, throughput is reduced strongly.
If a handover happens this step, throughput is reduced by interruption cost.

Simple meaning: if many users are attached to the same cell, each user gets a smaller share. If the signal is weak, the user needs more radio effort for the same demand, which increases cell pressure.

# 4. Data And Scenario Design

The project trains across multiple scenario families so the model does not memorize one map. The GNN is important because it sees relationships between cells rather than a fixed list of cell IDs.

| Scenario | Cells/UEs | Why it exists |
| --- | --- | --- |
| dense_urban | 20 cells, 160 UEs | Heavy city load with mixed mobility. |
| highway | 10 cells, 50 UEs | Fast movement along a road corridor; tests late handover. |
| highway_fast | 12 cells, 60 UEs | 80-120 km/h; forces proactive handover. |
| suburban | 15 cells, 105 UEs | Medium density and mixed movement. |
| sparse_rural | 7 cells, 25 UEs | Large cell spacing and limited alternatives. |
| overloaded_event | 12 cells, 156 UEs | Festival/stadium congestion, mostly static UEs. |
| real_pokhara | OpenCellID cells | Real cell layout generalization. |
| pokhara_dense_peakhour | Real Pokhara, high UE count | Real layout with peak-hour video-like demand. |
| kathmandu_real | 25-cell held-out test | Unseen dense real topology. |
| unknown_hex_grid | 19-cell held-out test | Unseen standard synthetic topology. |
| coverage_hole | 8-cell ring | Checks outage behavior when no target is good. |

## Feature Profiles

| Profile | Feature count | Use case | Important rule |
| --- | --- | --- | --- |
| ue_only | 11 | Main thesis path. Compatible with UE reports and drive-test style data. | RSRQ-derived load is a proxy only. PRB is not claimed. |
| oran_e2 | 15 | Future/demo path with network telemetry. | Adds true PRB utilization, PRB availability mask, connected UE count, and cell throughput. |

# 5. State, Action, Reward

## State: What The Model Sees

For one UE, the state is a small table with one row per candidate cell. In UE_ONLY, each row has 11 features. If there are 20 cells, the state shape is 20 x 11. The same model can work on different cell counts because the rows are connected by a graph.

| Feature | Plain meaning |
| --- | --- |
| rsrp_norm | How strong this cell sounds, normalized to 0-1. |
| rsrq_norm | How clean/loaded this cell sounds, normalized to 0-1. |
| rsrp_delta | How much stronger/weaker this cell is than the current serving cell. |
| rsrq_delta | How much better/worse quality is than the current serving cell. |
| rsrp_trend | Whether the signal is improving or getting worse since last step. |
| is_serving | 1 if this row is the current serving cell. |
| signal_usable | 1 if RSRP is above the minimum threshold. |
| ue_speed_class | How fast the UE is moving. |
| time_since_ho | How long since the last handover. |
| was_previous_serving | 1 if this was the previous serving cell, useful for ping-pong detection. |
| load_proxy_rsrq | Estimated load from RSRQ; this is not real PRB. |

## Action: What The Model Chooses

The action is an integer target cell. If the target equals the current serving cell, the UE stays. If the target is different, a handover is attempted. Invalid weak cells are masked so the model cannot intentionally select impossible targets.

For UE u:
state_u has shape (num_cells, feature_dim)
action a_u is one of: 0, 1, 2, ..., num_cells - 1
valid_action_mask[c] = True if cell c is usable, plus serving cell is always allowed

## Reward: What The Model Is Taught To Like

The reward is multi-objective. It rewards useful throughput, fairness, good load distribution, stable serving, proactive movement before outage, and escaping overloaded cells. It penalizes handover cost, recent repeated handovers, ping-pong, outage, weak targets, and moving into overloaded cells.

Reward is approximately:
+ throughput satisfaction
+ throughput improvement
+ P5 throughput improvement
+ Jain load fairness
+ load balancing improvement
+ proactive handover bonus
+ overloaded-cell escape bonus
+ stay bonus when staying is safe
- load imbalance
- overload
- handover cost
- recent-handover penalty
- ping-pong penalty
- outage and outage severity
- penalty for moving to a highly loaded target

Analogy: the reward is a report card. A handover gets good marks if it helps the user and the network, and bad marks if it creates instability or bad coverage.

# 6. Graph Construction

The GNN needs a graph. Each cell is a node. Edges connect nearby cells because handover decisions are mostly local: a UE normally moves between neighboring cells, not random distant cells.

For every pair of cells i and j:
d_ij = distance between cell positions
w_ij = exp(-d_ij / scale)

For each cell, keep the nearest graph_neighbors cells.
Symmetrize the graph so if i is linked to j, j is linked to i.

The model then passes messages between neighboring cells. In simple language, every cell row learns not just from its own features, but also from nearby cells. This is why it can generalize to a different topology like Kathmandu or a 40-cell layout.

# 7. Model Architecture

## High-Level Model

Input for one UE:
cell feature matrix X with shape (N cells, F features)
cell graph edges E with weights W

GNN layers:
GCN/GAT layer 1 -> LayerNorm -> ReLU/ELU -> Dropout
GCN/GAT layer 2 -> LayerNorm -> ReLU/ELU -> Dropout
GCN/GAT layer 3 -> LayerNorm -> ReLU/ELU

Dueling DQN head:
Value stream V(s)
Advantage stream A(s, cell)
Q(s, cell) = V(s) + A(s, cell) - mean_over_cells A(s, cell)

Output:
one Q-value per candidate cell

## Why GNN Instead Of A Plain Neural Network?

A plain neural network usually expects a fixed input size. If trained on 20 cells, it becomes awkward on 25 or 40 cells. A GNN handles each cell as a node and shares the same message-passing rule across all nodes. That means the same learned idea can move from Pokhara to Kathmandu-like topologies.

| Model choice | Strength | Weakness |
| --- | --- | --- |
| Flat DQN / MLP | Simple and fast for fixed-size experiments. | Tied to a fixed maximum cell layout; weaker topology generalization. |
| GNN-DQN | Learns relational handover logic across different cell graphs. | More complex and needs careful batching/masking. |
| SON-GNN-DQN | Keeps learned preference but deploys through bounded telecom parameters. | May be conservative because A3/TTT still gate the final handover. |

## Important Architecture Details

Default model uses GCN layers; GAT is available through config but the main path uses use_gat = false.

LayerNorm stabilizes training by keeping hidden activations in a manageable range.

Dropout is used during training but disabled for target inference and evaluation.

Dueling DQN separates “how good is this state overall?” from “which target cell is better?”

Double DQN reduces overestimation by selecting the next action with the online net but evaluating it with the target net.

# 8. Algorithm Reached

## Final Algorithm Name

The algorithm reached is best described as SON-GNN-DQN: a graph-based DQN preference learner plus a safety-bounded SON translation layer.

## Algorithm In Simple Steps

Build a graph from the cell topology.

For each UE, build a per-cell feature table from UE-observable measurements.

The GNN-DQN scores each candidate cell with a Q-value.

During direct GNN-DQN evaluation, the cell with highest valid Q-value is selected.

During deployable SON-GNN-DQN evaluation, the GNN preferences are aggregated by source-target cell pair.

The SON controller adjusts CIO values by small bounded steps, for example between -6 dB and +6 dB.

The actual handover still follows a standard A3 condition with TTT.

If throughput drops or ping-pong increases too much, SON can roll back the last CIO changes.

## SON Controller Math

Preference share:
pref_share(source,target) = number of UEs served by source that prefer target
                            / number of UEs served by source

If pref_share is high:
CIO(source,target) <- clip(CIO(source,target) + step, CIO_min, CIO_max)

If target appears overloaded:
CIO(source,target) <- clip(CIO(source,target) - step, CIO_min, CIO_max)

A3 with SON CIO:
RSRP_target + CIO(source,target) > RSRP_serving + base_A3_offset
and this must hold for TTT(source,target) steps.

This is safer than letting the neural network directly force all handovers. The model gives pressure; SON turns that pressure into familiar network knobs.

# 9. Training Pipeline

## Training Order

PYTHONPATH=src python3 -m pytest -q
python3 scripts/train.py --config configs/experiments/smoke_ue.json
python3 scripts/train.py --config configs/experiments/smoke_oran.json
python3 scripts/train.py --config configs/experiments/diagnostic_ue.json
python3 scripts/train.py --config configs/experiments/multiscenario_ue.json

## What Happens In One Training Episode

Pick one scenario, such as highway or dense_urban. Scenario sampling may be weighted.

Reset the simulator with a seed so UE positions, shadowing, speeds, and demands are initialized.

For every time step, advance UE mobility and update RSRP, RSRQ, load, and throughput snapshot.

Build states for all UEs in one batch.

Choose actions using epsilon-greedy exploration: sometimes random valid action, usually best Q-value.

Apply each UE action to the simulator, collect reward and next state.

Store transitions in the replay buffer for that scenario.

Every train_every decisions, sample a mini-batch and update the neural network.

Soft-update the target network with tau, or hard-update if tau is zero.

At the end, log metrics and optionally run held-out SON validation.

## DQN Update Equation

DQN learns by comparing the current Q-value with a target Q-value. If the current estimate is wrong, the network is adjusted.

n-step target:
y = r_0 + gamma*r_1 + ... + gamma^(n-1)*r_(n-1)
    + gamma^n * Q_target(next_state, best_next_action)

Double DQN best action:
best_next_action = argmax_a Q_online(next_state, a)

Loss:
loss = Huber(Q_online(state, action) - y)

Prioritized replay weight:
important transitions with larger TD error are sampled more often.

# 10. How Training Is Done In Batch

Batch training is one of the most important technical fixes. A single UE state is a graph with N cell nodes. A batch is many UE graphs at once. The code creates a larger disconnected graph by copying the same cell topology for each sample in the batch.

Single sample:
state shape = (N cells, F features)
edge_index shape = (2, E edges)

Batch of B samples:
states are flattened to (B*N, F)
edge_index is copied B times with node-index offsets:
  graph 0 uses nodes 0 ... N-1
  graph 1 uses nodes N ... 2N-1
  graph 2 uses nodes 2N ... 3N-1

The model is called with:
batch_size = B
nodes_per_graph = N

Output shape:
Q-values = (B, N)

## Why This Matters

Without explicit batch_size and nodes_per_graph, a 40-cell unseen topology might accidentally look like two 20-cell graphs. The code avoids that by requiring explicit batching information during training. For single-graph inference, it returns one Q-value vector of length N.

## Padding And Masks

Multi-scenario training has different cell counts. The agent is created with max_cells across training scenarios. Smaller scenarios are padded with zero rows, and invalid padded cells are masked out so they cannot be selected.

Example:
max_cells = 20
sparse_rural has 7 cells
state is padded from (7, 11) to (20, 11)
valid_mask is True for real valid cells and False for padded cells
Q-values for invalid cells are set to -infinity before argmax

# 11. Metrics, Benchmarks, And What The Comparisons Say

## Metrics

| Metric | Meaning | Why professors care |
| --- | --- | --- |
| avg_ue_throughput_mbps | Average per-user throughput. | Shows general user data-rate performance. |
| p5_ue_throughput_mbps | 5th percentile user throughput. | Shows worst-user protection, not just average. |
| total_throughput_mbps | Sum of all UE throughput. | Measures network-wide carried traffic. |
| load_std | Standard deviation of cell loads. | Lower means load is more balanced. |
| jain_load_fairness | Fairness of load distribution, closer to 1 is fairer. | Shows whether one or two cells are being overloaded. |
| outage_rate | Fraction of UEs below minimum serving signal. | Measures coverage failures. |
| overload_rate | Fraction of cells over capacity. | Shows congestion. |
| handovers_per_1000_decisions | How often the policy changes serving cells. | Too high means excessive signaling. |
| pingpong_rate | Fraction of handovers returning quickly to previous cell. | High ping-pong is unstable and bad for operators. |
| weak_target_ho_rate | Fraction of handovers to weak cells. | Should be near zero. |
| son_update_count | Number of CIO/TTT updates by SON. | Shows controller activity. |

## Benchmarks

| Method | Plain explanation | What it tests |
| --- | --- | --- |
| no_handover | UE never changes cell. | Checks whether handover is even needed. |
| random_valid | Randomly chooses among valid cells. | Checks if learning beats blind valid movement. |
| strongest_rsrp | Move to strongest signal if it clears hysteresis. | Classic signal-only baseline. |
| a3_ttt | Standard A3 with offset and time-to-trigger. | Main telecom baseline. |
| load_aware | Combines signal quality with RSRQ load proxy. | Rule-based load balancing baseline. |
| gnn_dqn | Raw learned target selection. | Research baseline for the learned preference model. |
| son_gnn_dqn | GNN preferences translated to bounded CIO/TTT. | Main deployable method. |

## 20-Seed Evaluation Summary

The following table averages the clean multiscenario_ue evaluation across the 11 available scenario CSVs. These are local generated results, not old pre-refactor archive outputs.

| Method | Avg Mbps | P5 Mbps | Load std | Jain | Outage | HO/1000 | Ping-pong |
| --- | --- | --- | --- | --- | --- | --- | --- |
| no_handover | 4.705 | 2.199 | 0.252 | 0.698 | 0.0036 | 0.0 | 0.000 |
| random_valid | 3.840 | 1.817 | 0.247 | 0.851 | 0.0125 | 910.5 | 0.093 |
| strongest_rsrp | 4.682 | 2.177 | 0.277 | 0.643 | 0.0019 | 24.0 | 0.190 |
| a3_ttt | 4.699 | 2.190 | 0.269 | 0.659 | 0.0026 | 9.8 | 0.000 |
| load_aware | 4.674 | 2.174 | 0.278 | 0.641 | 0.0023 | 25.8 | 0.239 |
| gnn_dqn | 4.331 | 1.846 | 0.500 | 0.561 | 0.0023 | 21.5 | 0.222 |
| son_gnn_dqn | 4.698 | 2.189 | 0.269 | 0.659 | 0.0026 | 9.9 | 0.000 |

## What The Comparison Says

The headline should be conservative and defense-ready: son_gnn_dqn is A3-competitive across training and held-out scenarios while maintaining near-zero ping-pong.

The direct raw gnn_dqn is not the main deployment result in these outputs. It is a learned preference baseline and input to the SON framing.

random_valid sometimes improves load fairness because random spreading can distribute UEs, but it causes extremely high handover rates and worse outage risk. It is not operationally acceptable.

In overloaded_event, random_valid can show higher average throughput by spreading users randomly. The honest interpretation is not “random is best”; it is “congestion stress exposes the tradeoff between throughput spreading and handover stability.”

A3 and SON-GNN-DQN are very close because the SON layer is intentionally bounded. That is a deployment strength: it changes standard parameters carefully instead of taking unsafe direct actions.

## Scenario-Level Message

| Scenario | SON avg Mbps | A3 avg Mbps | Random avg Mbps | SON ping-pong |
| --- | --- | --- | --- | --- |
| coverage_hole | 4.971 | 4.971 | 4.197 | 0.000 |
| dense_urban | 4.953 | 4.953 | 4.133 | 0.000 |
| dharan_synthetic | 5.072 | 5.072 | 4.166 | 0.000 |
| highway | 4.877 | 4.877 | 4.050 | 0.000 |
| kathmandu_real | 4.704 | 4.707 | 3.720 | 0.000 |
| overloaded_event | 5.131 | 5.136 | 5.808 | 0.000 |
| pokhara_dense_peakhour | 4.135 | 4.131 | 1.526 | 0.000 |
| real_pokhara | 4.978 | 4.977 | 3.924 | 0.001 |
| sparse_rural | 2.933 | 2.934 | 2.453 | 0.000 |
| suburban | 4.997 | 4.998 | 4.150 | 0.000 |
| unknown_hex_grid | 4.933 | 4.935 | 4.109 | 0.000 |

# 12. Checkpoints, Compatibility, And Acceptance Gates

## Checkpoint Metadata

Checkpoints store more than weights. They also store feature profile, feature dimension, max cells, training config, scenario names, model version, reward version, and training state. This is important because loading a UE-only checkpoint into an O-RAN feature profile would silently break the meaning of input features.

## Compatibility Rules

Resume only with the same config and scenario list.

Feature profile must match: ue_only checkpoint should not load as oran_e2.

Feature dimension must match: UE_ONLY uses 11 features, ORAN_E2 uses 15.

Resume checkpoints may include replay; full replay is large, so it is opt-in.

Old pre-refactor outputs are diagnostic only and should not be cited as final results.

## Acceptance Gate Before Full Training

| Gate | Why it matters |
| --- | --- |
| Unit/integration tests pass. | Confirms simulator, model shapes, metrics, and policies are not broken. |
| UE_ONLY and ORAN_E2 smoke runs finish without NaNs or invalid actions. | Confirms both feature profiles are technically usable. |
| Kathmandu-style 25-cell and larger 40-cell inference work. | Confirms topology generalization and batching are safe. |
| Checkpoint reload rejects incompatible feature profiles. | Prevents invalid comparisons. |
| random_valid baseline is present. | Stops weak comparisons that only beat easy baselines. |
| Short diagnostic GNN-DQN beats random before long run. | Prevents wasting time on a broken learner. |
| Normal urban/suburban/highway scenarios do not show artificial outage. | Checks coordinate normalization and topology setup. |

# 13. Examples

## Example 1: Slow UE In A City

A UE is walking in dense_urban. The current cell has decent RSRP but poor RSRQ because many users are attached. A nearby cell has slightly weaker RSRP but better RSRQ. A strongest-RSRP rule may stay. A load-aware learner may prefer the less crowded neighbor if the signal remains usable. SON-GNN-DQN may increase the CIO toward that neighbor slightly, but A3/TTT still requires the condition to stay stable before handover.

## Example 2: Fast UE On Highway

A UE is moving quickly away from the serving cell. The serving signal is still usable but falling. The target cell is only a little better right now. The reward gives a proactive bonus if the handover prevents later outage, and the handover penalty is lower for fast UEs. This teaches the model that fast highway users should sometimes move earlier than slow city users.

## Example 3: Ping-Pong

A UE moves from cell A to cell B and quickly returns to A within the ping-pong window. The simulator counts this as ping-pong and applies a strong penalty. The SON controller may also increase TTT when ping-pong is high, meaning the network waits longer before allowing another handover.

# 14. Training And Evaluation Commands

## Recommended Sequence

# Tests
PYTHONPATH=src python3 -m pytest -q

# Smoke runs
python3 scripts/train.py --config configs/experiments/smoke_ue.json
python3 scripts/train.py --config configs/experiments/smoke_oran.json

# Diagnostic and main UE-only run
python3 scripts/train.py --config configs/experiments/diagnostic_ue.json
python3 scripts/train.py --config configs/experiments/multiscenario_ue.json

# Evaluate saved checkpoint over 20 seeds
python3 scripts/evaluate.py   --checkpoint results/runs/multiscenario_ue/checkpoints/gnn_dqn.pt   --out-dir results/runs/multiscenario_ue/eval_20seed   --seeds 20

## Resume Example

python3 scripts/train.py   --config configs/experiments/multiscenario_ue.json   --resume results/runs/multiscenario_ue/checkpoints/resume/resume_ep0025.pt

Do not resume with a different config. That makes the run scientifically hard to defend.

# 15. Defense Narrative

## What To Claim

This is a SON-compatible handover optimization framework, not a live O-RAN xApp.

The main feature profile is UE_ONLY, so it matches drive-test and UE measurement-report data.

RSRQ is used as a UE-observable load/interference proxy, not as real PRB utilization.

The GNN supports different cell counts and topologies because it operates on graph nodes and edges.

The SON layer converts learned preferences into bounded CIO/TTT updates, which is safer and closer to deployable telecom practice.

O-RAN/E2 can later add real PRB utilization and network KPIs without changing the main idea.

## What Not To Overclaim

Do not say the system is a deployed real-time O-RAN xApp.

Do not say RSRQ proxy is actual PRB utilization.

Do not cite pre-refactor archived results as final evidence.

Do not headline raw gnn_dqn if son_gnn_dqn is the deployable method.

Do not claim universal throughput dominance. Say the method is stable, topology-generalized, and A3-competitive, with a clear path for stronger load-aware gains.

# 16. Professor Defense Questions And Simple Answers

## Conceptual Questions

### Q: Why use a GNN?

A: Because cellular networks are naturally graphs. Cells are nodes and nearby cells are edges. A GNN can learn a rule that works on different topologies, instead of memorizing one fixed cell layout.

### Q: Why use DQN?

A: The action is discrete: choose one target cell or stay. DQN is a natural fit for discrete action-value learning.

### Q: Why not directly deploy raw GNN-DQN?

A: Direct neural handover can be unsafe. The defense-ready path lets GNN-DQN produce preferences, then a bounded SON controller translates them to standard CIO/TTT parameters.

### Q: What is the novelty?

A: The combined framing: UE-observable GNN-DQN handover preferences, topology generalization, explicit batching for unseen cell counts, and SON-safe translation into A3/CIO/TTT updates.

### Q: Is this O-RAN?

A: Not as the main claim. It is SON-compatible today. O-RAN/E2 is a future/demo extension when true PRB and KPM counters are available.

### Q: What does UE_ONLY mean?

A: The model uses measurements that a UE or drive-test can observe: RSRP, RSRQ, trends, serving state, mobility, and handover history. It does not require true PRB.

### Q: Why is RSRQ used?

A: RSRQ reflects signal quality affected by interference and load. It is useful as a load/interference proxy, but it is not claimed as real PRB utilization.

### Q: What is ping-pong?

A: A UE hands over from A to B and quickly returns to A. It causes signaling overhead and instability, so the reward and metrics penalize it.

### Q: Why include random_valid?

A: It is a sanity baseline. If a learned method cannot beat random valid actions, the training is not trustworthy.

### Q: Why can random sometimes have high throughput?

A: Random spreading can accidentally distribute UEs away from crowded cells, but it causes huge handover rates and instability, making it unsuitable for operators.

## Technical Questions

### Q: What is the input shape?

A: For one UE, the input is (num_cells, feature_dim). UE_ONLY has feature_dim = 11. During batch training, B samples become (B*num_cells, feature_dim).

### Q: What is the output?

A: One Q-value per candidate cell. The selected action is the valid cell with the highest Q-value.

### Q: How are invalid actions prevented?

A: The valid action mask marks weak or padded cells as invalid. Their Q-values are set to negative infinity before argmax.

### Q: What is the Bellman target?

A: y = reward + gamma^n * Q_target(next_state, best_next_action), with Double DQN selecting the best action by the online network.

### Q: What is prioritized replay?

A: Transitions with larger TD errors are sampled more often so rare important events, such as handovers or outage cases, are revisited during training.

### Q: What is n-step return?

A: Instead of learning only from the immediate reward, the model sums rewards across n future steps, improving credit assignment.

### Q: Why use a target network?

A: It stabilizes DQN training by keeping the target estimate slower-moving than the online network.

### Q: Why use LayerNorm and gradient clipping?

A: They reduce unstable updates and help avoid exploding activations or gradients.

### Q: How does batching handle different topology sizes?

A: Training pads smaller scenarios to max_cells and uses masks. The GNN forward call explicitly receives batch_size and nodes_per_graph to prevent topology-size confusion.

### Q: How does SON use GNN output?

A: It counts UE preferences by source-target cell pair. If enough UEs prefer a target, SON gently adjusts CIO toward that target within safe limits.

### Q: How is rollback handled?

A: If throughput drops too much or ping-pong increases beyond configured thresholds, the SON controller restores previous CIO values.

### Q: What is the reward trying to optimize?

A: Throughput satisfaction, P5 throughput, load fairness, proactive handover, low outage, low overload, low ping-pong, and controlled handover rate.

## Results Questions

### Q: What is the main result?

A: SON-GNN-DQN is A3-competitive across the evaluated scenarios and maintains near-zero ping-pong, while preserving a safer deployment story.

### Q: Does it beat A3 everywhere?

A: Not strongly in the current 20-seed table. It mostly matches A3. The honest claim is stable SON-compatible learning, not universal throughput dominance.

### Q: Why is raw gnn_dqn weaker in the summary?

A: Raw direct target selection is a research baseline. The deployable method is the SON layer using learned preferences. Raw GNN-DQN can be more volatile without A3/TTT safety.

### Q: What does P5 throughput show?

A: It shows the lower-tail users. High average throughput can hide suffering users; P5 reveals whether the worst users are protected.

### Q: Why include held-out scenarios?

A: To show topology generalization. Kathmandu, Dharan, unknown hex grid, and coverage hole are not simply the same training map repeated.

### Q: What are final result tables required to include?

A: no_handover, random_valid, strongest_rsrp, a3_ttt, load_aware, gnn_dqn, and son_gnn_dqn.

# 17. Limitations And Future Work

The current main path uses a simulator and local generated scenarios. Real field deployment would need live network integration and operator approval.

UE_ONLY cannot know true PRB utilization. It uses RSRQ as a proxy, which is useful but imperfect.

The current SON layer is intentionally conservative, so it may match A3 more than dramatically outperform it.

O-RAN/E2 future work can add true PRB, cell throughput counters, and near-RT RIC integration.

More extensive final defense runs should use the long defense config with behavioral cloning, PER, n-step returns, held-out validation, and replay-inclusive resumes.

# 18. One-Page Final Summary

The project creates a GNN-DQN model for handover preference learning and wraps it with a safety-bounded SON controller. The model sees UE-observable measurements for every candidate cell, scores possible target cells, and learns from rewards that combine throughput, fairness, load, outage, and stability. The GNN lets the same model reason over different cell layouts. Training uses multi-scenario replay, target networks, Double DQN, optional prioritized replay, n-step returns, batched GNN updates, exploration decay, and checkpoint validation. The final defense framing is SON-GNN-DQN: learned preferences are translated into bounded CIO/TTT updates and then executed through standard A3-style logic. This makes the system more realistic and defensible than a raw neural network directly controlling handovers.

Defense-safe closing sentence Our contribution is not just “AI chooses a cell.” It is a topology-generalized learned preference model connected to a safety-bounded SON layer that speaks the language of real handover parameters.